PRODUCT REQUIREMENTS DOCUMENT (PRD)
Raspberry Pi 4B & Portronics Webcam Pothole Detection System

Edge Computer Vision Implementation Based on Custom Pothole Taxonomy Dataset

1. Document Overview & Project Scope

This Product Requirements Document (PRD) outlines the specifications for building a low-cost, edge-based pothole detection system utilizing a Raspberry Pi 4B single-board computer paired with a standard Portronics USB webcam. The computer vision model and classification logic are trained and structured around our custom roadway surface dataset file: “Pothole_detection_dataset.xlsx” (39 annotated entries covering five core defect categories: Disintegrated, Manhole, Water clogged, Raveling, and Deformation across various severity levels).

2. System Architecture & Bill of Materials

Compute Unit: Raspberry Pi 4B (4GB RAM model) running Raspberry Pi OS (64-bit Debian Bookworm).

Imaging Sensor: Portronics Standard USB Webcam (plug-and-play UVC protocol, 720p/1080p resolution capture via USB 2.0/3.0 port).

Power Supply: Official Raspberry Pi 5V/3A USB-C Power Supply or regulated 12V-to-5V vehicle buck converter.

Storage: 32GB+ Class 10 / UHS-1 MicroSD card for OS, Python environment, and model weights.

Software Stack: Python 3.9+, OpenCV (cv2), NumPy, TensorFlow Lite Runtime (TFLite) or ONNX Runtime for optimized CPU inference.

3. Dataset Taxonomy & Classification Rules

The machine learning and heuristic pipeline is structured around the five key categories defined in our dataset analysis:

Disintegrated Surfaces (16 instances): Includes shallow surface potholes, bowl-shaped potholes, and sudden deep structural failures. Severity ranges from low to high.

Manhole Anomalies (8 instances): Includes sunken manholes, crumbled collars, broken open lids, and deteriorated metal surrounds. Severity: medium to highest.

Water Clogged Hazards (6 instances): Includes standard and severe water-filled depressions that mask underlying depth. Severity: medium to high.

Raveling Surfaces (5 instances): Includes shallow surface raveling, cracked aggregates, and widespread material loss. Severity: low to medium.

Deformation & Trenches (4 instances): Includes trench potholes, deep longitudinal cuts, and hyper-deep trenches. Severity: medium to high.

4. Functional Pipeline & Processing Logic

Frame Capture: OpenCV initializes the Portronics webcam at 640x480 resolution @ 30 FPS to maintain low latency on the Raspberry Pi 4 CPU.

Region of Interest (ROI) Masking: Crops the upper 40% of the frame (sky/horizon) to focus computational power exclusively on the road surface ahead.

Model Inference: A quantized YOLOv8n-tflite or MobileNetV2 classifier processes each frame to categorize defect type (Disintegrated, Manhole, Water Clogged, etc.) and assign a severity score (Low, Medium, High).

Local Logging & Alerting: Detections are timestamped and logged locally to a CSV file along with categorical metadata, allowing post-survey inspection.

5. Implementation Roadmap

Phase 1: Environment Setup: Flash Raspberry Pi OS, configure USB webcam permissions, and install OpenCV and TFLite libraries.

Phase 2: Model Training & Conversion: Train custom YOLOv8 model on the dataset categories and export to `.tflite` format.

Phase 3: Python Script Deployment: Deploy real-time inference script with ROI masking and CSV logging.

Phase 4: Field Testing: Mount webcam on vehicle dashboard, calibrate height/angle, and execute test runs across target roads.